ETICD Lab#1

We have been asked to use simulations to find different parameters of a Nmos transistor and Pmos transistor, and simulate a inverter using Nmos and Pmos transistors.

- Nmos

The Nmos transistor has to follow the following specifications;  W/L=1μm/200nm and have a Vdd=1.5V. Below is the schematic of the transistor used in our simulations.

Figur 1 NMOS test bench schematic

Subthreshold slope

Figur 3 Subthreshold slope of the NMOS transistor

The subthreshold slope is fund be looking at the graph (Figur 2) in the subthreshold region where Vgs < Vth. To find the subthreshold you can measure the voltage difference between two decades. So, for our simulation the subthreshold slope is 252mV-169mV = 83mV/decade. The typical subthreshold at room temperature is around 60mV/decade.

DIBL

Figur 4 DIBL of the NMOS transistor with markers for the two threshold voltages

The DIBL(Drain-Induced Barrier Lowering) increases the drain source voltage leakage in transistors with short channels at low Vds voltage. By lowering the Vds voltage on the transistor and plot sweep on the same graph as your normal Vds. You can find the delta Vth and delta Vds and calculate the DIBL effect be this formula  . For our transistor (see Figur 3), the DIBL is

Ligningen løses for DIBL vha. CAS-værktøjet WordMat.

Ioff

Figur 5 Ioff shown on the graph

The Ioff is the current leakage from drain to source when the gate voltage is equal to zero and the NMOS transistor is turned off. As seen in Figur 4 the Ioff for our simulated NMOS transistor is ~12pA.

Threshold Voltage

Figur 6 Threshold voltage of the NMOS transistor

To read a transistor threshold voltage from a linear graph. you draw a line from the straight part of the graph as seen on Figur 5. Where the line crosses the x axis you have your threshold voltage. In our case the Vth is ~539mV.

Temperatur effects

Figur 7 Temperature effect on a NMOS transistor shown with 5 graphs between -40 and 125 degrees, blue line is at -40 degrees and the red line is at 125 degrees

In the temperature sweep of the NMOS transistor with Vgs=0. You can see in the graph on Figur 6 that the Ioff increases when the temperature raises. Also we see that the largest effect the temperature has is in the region around Vt, where the spread is the largest between the temparatures before the saturation region.

Body voltage effect on threshold voltage

Figur 8 Threshold voltage increased after the Body is connected to Vdd instead of gnd

By increasing the body voltage we can manipulate the threshold voltage of the transistor. As seen in Figur 9 the threshold voltage is increased to 860mVcompared to the ~560 we had before, when the bulk gate is connected to Vdd instead of gnd.

Channel length modulation coefficient

Figur 9 Graph of the Nmos transistors Ids-Vds characteristics

To find the Channel length modulation coefficient we follow the equation I_dsat/R_on, both of these can be found on the graph. I_dsat is the current when the transistor enters the saturation region, read as the current when the voltage is at Vt. R_on is the slope of the linear region after the transistor enters the saturation region. Reading the graph and substituting into our equation we get a Channel length modulation coefficient of: 0.118

- Pmos

The Pmos transistor follows the specificationsas the Nmos;  W/L=1μm/200nm and have a Vdd=1.5V. Below is the schematic of the transistor used in our simulations. for many of the parameters, the same method used for a Nmos transistor applies to the Pmos transistor as well, in these cases we will simply write the final equation or value, while refering to the Nmos section for explanations.

Figur 10 Schematic of the Pmos transistor

Subthreshold slope

Figur 11 Subthreshold slope of the PMOS transistor

The subthreshold slope of the Pmos is(see Nmos scetion for definition): 90mV/Dec

DIBL

The DIBL(Drain-Induced Barrier Lowering) of the Pmos is(see Nmos scetion for definition):: 16.9mV

Ioff

The Ioff current of the Pmos is(see Nmos scetion for definition): -5.33pA

Threshold Voltage

Figur 12 Threshold voltage of the PMOS transistor

reading the threshold voltage is a bit different for the Pmos graph, here you need to find the point the graph enters the saturation region, or more specifik where the graph becomes linear and read the voltage value at this point. We read the Threshold voltage to be: 0.49 V

Temperatur effects

Figur 12 Temperature effect on a PMOS transistor shown with 5 graphs between -40 and 125 degrees, blue line is at -40 degrees and the red line is at 125 degrees

Again this graph looks different from the Nmos graph, however it shows the same effect and tendencies in increasing the spread around Vt after changing the temperature.

Body voltage effect on threshold voltage

Figur 13 Threshold voltage increased after the Body is connected to Vdd instead of gnd

By increasing the body voltage we can manipulate the threshold voltage of the transistor. As seen in Figur 13 the threshold voltage is increased to 800mVcompared to the ~560 we had before, when the bulk gate is connected to Vdd instead of gnd.

Channel length modulation coefficient

Figur 14 Graph of the Nmos transistors Ids-Vds characteristics

To find the Channel length modulation coefficient we follow the equation I_dsat/R_on, both of these can be found on the graph. I_dsat is the current when the transistor enters the saturation region, read as the current when the voltage is at Vt. R_on is the slope of the linear region after the transistor enters the saturation region. Reading the graph and substituting into our equation we get a Channel length modulation coefficient of:

- Inverter

The last part of our assignment is simulating a dc-sweep over a inverter. We have used the following schematic for our inverter

Figur 15 Schematic of inverter in Virtuoso

Sizing the transistors

first we have to size the transistors to get the best noise-margin

Figur 16 graph of V(in)/V(out) for inverter without sizing(1 to 1 ratio)

Figur 17 graph of V(in)/V(out) for inverter with sizing(1 to 3 ratio)

In order to get the best noise margin, we need to have V_IH and V_IL(see figure 18) to be closer to eachother. We can see that after resizing the transistors that our graph tends towards this ideal.

Figur 17 graph of V(in)/V(out) for CMOS taken from "CMOS VLSI design, page 92)

Temperature effect on dc-values of inverter

Figur 18 graph of V(in)/V(out) for inverter the a range of temperatures

in figur 18 we can see that as the temperature rises the point where the input crosses the output at vdd/2 is moved, this is better seen in figur 19 where we have zoomed in on the region around this point

Figur 19 zoom in of figur 19 around the VDD/2 point(dark blue line is 27 degrees)

in figur 19 we can more clearly see that the temperature swings depending on the temperature.